







  Distr.PRIVATE 









  GENERAL









  UNEP/OzL.Pro/WG.1/11/3









  29 December 1994









  ORIGINAL:  ENGLISH

OPEN-ENDED WORKING GROUP OF THE

  PARTIES TO THE MONTREAL PROTOCOL

Eleventh meeting

Nairobi, 8-12 May 1995


THE 1994 SCIENCE, ENVIRONMENTAL EFFECTS, AND TECHNOLOGY AND

 
ECONOMIC ASSESSMENTS


Synthesis Report*

*
Prepared at the request of the United Nations Environment Programme, on behalf of the Parties to the Montreal Protocol.

Na.94-5003   220195
/...


Synthesis Report Panel
Daniel L. Albritton

United States National Oceanic and Atmospheric Administration

Stephen O. Andersen

United States Environmental Protection Agency

Piet J. Aucamp

South Africa Department of Health

Suely Carvalho

University of Sao Paulo, Brazil

Lambert Kuijpers

Technical University, Eindhoven, Netherlands

K. M. Sarma
Ozone Secretariat, United Nations Environment Programme, Nairobi

Xiaoyan Tang

Peking University, China

Manfred Tevini

University of Karlsruhe, Germany

Jan C. van der Leun

University Hospital, Utrecht, Netherlands

Robert T. Watson
United States Office of Science and Technology Policy


CONTENTS

Chapter








Paras
Page
EXECUTIVE SUMMARY





  ES.1 - ES.15  
 4

PREFACE








  1-3
 7

I.
MAJOR RECENT FINDINGS




  4
 9


A.
Ozone science findings



  4
 9


B.
Environmental effects findings 


  5-13 
 13


C.
Technology and economics findings 

 14-21
 14

II.
FEASIBILITY OF OPTIONS FOR LOWERING


STRATOSPHERIC CHLORINE AND BROMINE ABUNDANCES
  22-23 
 16


A.
Approaches to lowering stratospheric



chlorine and bromine abundance that 



are technically and economically



feasible





  24-28 
 17



1.
Further control of methyl bromide

  24-25 
 17



2.
Reductions in the HCFC cap and




acceleration of the phase-out




schedule 





  26-28 
 17


B.
Approaches to lowering stratospheric 



chlorine and bromine abundances that



are not technically and/or



economically feasible 



  29-34 
 18



1.
Recover and destroy halon


  29-32 
 18



2.
Recover and destroy CFCs


  33-34 
 19

III.
FUTURE ASSESSMENT WORK:  HCFC AND METHYL


BROMIDE ELABORATION (DECISION VI/13),


CHALLENGES FACING ARTICLE 5 COUNTRIES AND

 
COUNTRIES WITH ECONOMIES IN TRANSITION AND


ESSENTIAL USES 





  35-37
 19


Appendix:  LIST OF SCIENCE, ENVIRONMENTAL


EFFECTS, AND TECHNOLOGY AND ECONOMIC


ASSESSMENT EXPERTS 
 20


1994 SYNTHESIS REPORT

EXECUTIVE SUMMARY
ES.I.
The rates of build-up in the atmosphere of human-made compounds that deplete the ozone layer (e.g. chlorofluorocarbons (CFCs) and halons) have slowed in recent years as a direct result of reductions in global emissions of these compounds, thus demonstrating the intended impact of the Montreal Protocol and its Amendments and Adjustments.

ES.2.
The peak global ozone depletion is expected to occur during the next several years, and the stratospheric ozone layer is expected to recover (if other factors remain unchanged) in about 50 years in response to international actions under the Montreal Protocol and its Amendments and Adjustments.

ES.3.
Unusually low global ozone has been observed over the past two years, with the most severe Antarctic ozone "hole" and lowest seasonally averaged ozone in the northern hemisphere.  These anomalies are likely due, in part, to chemical processes linked to the 1991 eruption of Mt. Pinatubo in the Philippines.  The 1994 global ozone levels are returning to values closer to those expected from the longer-term downward trend.

ES.4.
Methyl bromide continues to be viewed as a significant ozone-depleting chemical (with an ozone-depleting potential (ODP) of 0.6).  Three major anthropogenic sources of methyl bromide are identified:  agricultural usage (soil and commodity fumigation), biomass burning, and potentially the exhaust of automobiles using leaded gasoline; in addition to the natural ocean source.  

ES.5.
The link between a decrease in stratospheric ozone and an increase in surface ultraviolet (UV) radiation has been further strengthened.  Measurements of UV radiation at the surface under clear‑sky conditions show that low overhead ozone yields high UV radiation and in the amount predicted by radiative‑transfer theory.  Large increases of surface UV are observed in Antarctica and the southern part of South America during the period of the seasonal ozone "hole."  

ES.6.
Increases in UV-B radiation (290-315mm) have substantial adverse effects on human health (skin cancer, eye disease, immune systems), and animals, terrestrial plants, aquatic organisms, biogeochemical cycles, tropospheric air quality, and materials.  It is estimated that a sustained 1% decrease in stratospheric ozone will result in an increase in non-melanoma skin cancer incidence of approximately 2%.  One study has indicated a 6-12% reduction in phytoplankton production in the marginal ice zone during the period of peak spring-time Antarctic ozone depletion.

ES.7.
Developed countries, except for a number of countries with economies in transition (CEITs), are generally on schedule to phase out CFCs, carbon tetrachloride, and 1,1,1-trichloroethane (methyl chloroform) by 1996.  Halon production was phased out by 1 January 1994.

ES.8.
It seems inevitable that compliance of several countries with economies in transition (CEITs) will not occur in 1996 and that significant efforts will be necessary for eventual compliance.

ES.9.
In developed countries, the most difficult remaining challenges to the phase-out are for refrigeration and air conditioning servicing, 1,1,1-trichloroethane solvent use among small and medium-sized industry, metered dose inhalers, precision cleaning of sophisticated aerospace equipment (rocket motors, oxygen systems, and deep-space guidance systems), and for laboratory and analytical applications.

ES.10.
Implementation of alternatives and substitutes to methyl bromide is proceeding in several non-Article 5 countries and is virtually complete in the Netherlands, with the exception of some quarantine uses.  Because alternatives to methyl bromide are technically the same in all countries, phase-out schedules for Article 5 countries could be similar to non-Article 5 countries, provided that adequate financing and training are available.

ES.11.
The essential use nomination process was successful and collaborative.  In many cases, experts identified suitable alternatives or substitutes or helped guide applicants through rapid identification and development.

ES.12.
A new, and apparently increasing, challenge is the smuggling of newly produced CFCs and halon.  Measures that could reduce this illegal activity could be designed in a way to ensure that quantities and procedures are legitimate, rather than to prohibit trade.

ES.13.
Many developing countries are making progress in the phase-out of ozone-depleting substances (ODSs) in a variety of application areas, but are concerned about the availability of ozone-depleting substances, the need for a sustained commitment to assist developing countries, the adequacy of support for the Multilateral Fund for the Implementation of the Montreal Protocol, the capacity of developing countries to adopt new technologies, and the barriers to information exchange.

ES.14.
Hydrochloroflurocarbons (HCFCs) remain critical for meeting the near-term CFC phase-out goals.  However, they are less important for new equipment produced in the mid- and long-term period.  HCFCs are currently necessary for certain new refrigeration and air-conditioning applications, for servicing already installed HCFC equipment, for some rigid thermal insulating and automotive safety foam products, and for several important small uses such as sterilization and precision cleaning.  It is not yet determined whether HCFCs will be required to replace halon in critical uses.

ES.15.
There are only a limited number of approaches to lowering stratospheric chlorine and bromine abundances beyond those already adopted by the Parties to the Protocol.  Four approaches identified by the Science Assessment Panel were evaluated by the Technology and Economics Assessment Panel:

PRIVATE 
Possible additional controls
Conclusion

1.  Methyl bromide reductions
Reductions in some methyl bromide uses for fumigation are technically and economically feasible, with further analysis to be provided in the March 1995 Assessment Report to Parties



2.  HCFC phase-out schedule
Further controls of HCFCs are technically and economically feasible, with further analysis to be provided in the March 1995 Assessment Report to the Parties



3.  Halon destruction
Although it is technically feasible to destroy halon, existing halon stocks are required for critical uses that have no identified substitutes or alternatives at present



4.  CFC destruction
Although it is technically feasible to destroy CFCs, it is not economically feasible because CFC is required for servicing existing refrigeration and air-conditioning equipment at present




1994 SYNTHESIS REPORT


PREFACE
1.
This document is a synthesis by the Assessment Panel Co-Chairs of the latest Science, Environmental Effects, and Technology and Economic Assessment Panel reports
 that will be part of the information upon which the Parties to the Montreal Protocol will base future decisions regarding protection of the stratospheric ozone layer.  Full assessment reports are available from the Ozone Secretariat.

2.
Specifically, the Montreal Protocol on Substances that Deplete the Ozone Layer states in its Article 6 that "the Parties shall assess the control measures ... on the basis of available scientific, environmental, technical, and economic information".  To provide the mechanisms whereby these assessments are conducted, the Protocol further states that "the Parties shall convene appropriate panels of experts ..." and "... the panels will report their conclusions ... to the Parties".

3.
The 1994 assessment reports were prepared by a total of more than 700 of the world's leading experts from 46 countries:  Argentina, Australia, Austria, Bahamas, Bangladesh, Belgium, Benin, Brazil, Canada, Czech Republic, Chile, China, Cuba, Denmark, Egypt, France, Germany, Greece, Hungary, India, Iran (Islamic Republic of), Ireland, Israel, Italy, Japan, Jordan, Kenya, Malaysia, Mexico, Netherlands, New Zealand, Norway, Poland, Russian Federation, Singapore, Republic of South Africa, Spain, Saudi Arabia, Sweden, Switzerland, Syrian Arab Republic, Thailand, United Kingdom, United States of America, Venezuela, and Zimbabwe.  The chronology of those assessments and the relation to the international policy process are summarized on table 1.

Table 1
CHRONOLOGY OF ASSESSMENTS AND INTERNATIONAL POLICY  

Year
Policy Process


Assessments
1981





The Stratosphere 1981 Theory and Measurements 

1985
Vienna Convention
Atmospheric Ozone 1985
1987
Montreal Protocol

1988





Report of the International Ozone Trends Panel  

1989





Scientific, environmental effects, technology, and economics
 assessments of stratospheric ozone:  1989

1990
London Amendment

1991





Scientific, environmental effects, and technology and economics assessments of ozone depletion:  1991

1992





Methyl Bromide:  Its Atmospheric Science, Technology, and Economics (Assessment Supplement) 

1992
Copenhagen Amendment

1994





Technology and Economic Assessment Report, Including Recommendations on Nominations for Essential Use Production/Consumption Exemptions for Ozone-Depleting Substances, March 1994; 1994 scientific, environmental effects, and technology and economics assessment reports on ozone depletion

1995





Assessment report, including further elaboration of methyl bromide and HCFC control options,
 March 1995   


I.  MAJOR RECENT FINDINGS

A.  Ozone science findings
4.
The laboratory investigations, atmospheric observations, and theoretical and modelling studies of the past few years have provided a deeper understanding of the human‑influenced and natural chemical changes in the atmosphere and their relation to the Earth's stratospheric ozone layer and radiative balance of the climate system.  Since the last international scientific assessment of the state of understanding, there have been several key ozone‑related findings, observations, and conclusions:


(a)
The atmospheric growth rates of several major ozone‑depleting substances have slowed, demonstrating the intended impact of the Montreal Protocol and its Amendments and Adjustments.  The abundances of the chlorofluorocarbons (CFCs), carbon tetrachloride, methyl chloroform, and halons in the atmosphere have been monitored at global ground‑based sites since about 1978.  Over much of that period, the annual growth rates of these gases have been positive.  However, the data of recent years clearly show that the growth rates of CFC‑11, CFC‑12, halon‑1301, and halon‑1211 are slowing down.  The abundance of carbon tetrachloride is actually decreasing.  The observed trends in total tropospheric organic chlorine are consistent with reported production data, suggesting less emission than the maximum allowed under the Montreal Protocol and its Amendments and Adjustments.  Peak total chlorine/bromine loading in the troposphere is expected to occur in 1994, but the stratospheric peak will lag by about 3‑5 years.  Since the stratospheric abundances of chlorine and bromine are expected to continue to grow for a few more years, increasing global ozone losses are predicted (other things being equal) for the remainder of the decade, with gradual recovery in the twenty-first century;


(b)
The atmospheric abundances of several of the CFC substitutes are increasing, as anticipated.  With phase-out dates for the CFCs and other ozone‑depleting substances now fixed by international agreements, several hydrochlorofluorocarbons (HCFCs) and hydrofluorocarbons (HFCs) are being manufactured and used as substitutes.  The atmospheric growth of some of these compounds (e.g., HCFC‑22) has been observed for several years, and the growth rates of others (e.g., HCFC‑142b and HCFC‑141b) are now being monitored;


(c)
Record low global ozone levels were measured over the past two years.  Anomalous ozone decreases were observed in the mid-latitudes of both hemispheres in 1992 and 1993.  The northern hemispheric decreases were larger than those in the southern hemisphere.  Globally, ozone values were 1‑2% lower than would be expected from an extrapolation of the trend prior to 1991, allowing for solar‑cycle and quasi‑biennial‑oscillation (QBO) effects.  The 1994 global ozone levels are returning to values closer to those expected from the longer‑term downward trend;


(d)
The stratosphere was perturbed by a major volcanic eruption.  The eruption of Mt. Pinatubo in 1991 led to a large increase in sulphate aerosol in the lower stratosphere throughout the globe.  Reactions on sulphate aerosols resulted in significant, but temporary, changes in the chemical partitioning that accelerated the photochemical ozone loss associated with reactive hydrogen (HOx), chlorine, and bromine compounds in the lower stratosphere in the mid-latitudes and polar regions.  The observed 1994 recovery of global ozone is qualitatively consistent with observed gradual reductions of the abundances of these volcanic particles in the stratosphere;


(e)
Downward trends in total‑column ozone continue to be observed over much of the globe, but their magnitudes are underestimated by numerical models.  Over the period 1979-1994, decreases in ozone abundances of about 4‑5% per decade at mid-latitudes in the northern and southern hemispheres were observed by both ground‑based and satellite‑borne monitoring instruments.  At mid-latitudes, the losses continue to be much larger during winter/spring than during summer/autumn in both hemispheres, and the depletion increases with latitude, particularly in the southern hemisphere.  Little or no downward trends are observed in the tropics (20(N ‑ 20(S).  While the current two‑dimensional stratospheric models simulate the observed trends quite well during some seasons and at some latitudes, they underestimate the trends by factors of up to three in winter/spring at mid‑ and high-latitudes.  Several known atmospheric processes that involve chlorine and bromine and that affect ozone in the lower stratosphere are difficult to model and have not been adequately incorporated into these models;  


(f)
Observations have demonstrated that halogen chemistry plays a larger role in the chemical destruction of ozone in the mid-latitude lower stratosphere than expected from gas phase chemistry.  Direct in situ measurements of radical species in the lower stratosphere, coupled with model calculations, have quantitatively shown that the in situ photochemical loss of ozone due to (largely natural) reactive nitrogen (NOx) compounds is smaller than that predicted from gas-phase chemistry, while that due to (largely natural) HOx compounds and (largely anthropogenic) chlorine and bromine compounds is larger than that predicted from gas-phase chemistry.  This confirms the key role of chemical reactions on sulphate aerosols in controlling the chemical balance of the lower stratosphere.  These and other recent scientific findings strengthen the conclusion of the previous assessment that the weight of scientific evidence suggests that the observed middle‑ and high‑latitude ozone losses are largely due to anthropogenic chlorine and bromine compounds;


(g)
The conclusion that anthropogenic chlorine and bromine compounds, coupled with surface chemistry on natural polar stratospheric particles, are the cause of polar ozone depletion has been further strengthened.  Laboratory studies have provided a greatly improved understanding of how the chemistry on the surfaces of ice, nitrate, and sulphate particles can increase the abundance of ozone‑depleting forms of chlorine in the polar stratosphere.  Furthermore, satellite and in situ observations of the abundances of reactive nitrogen and chlorine compounds have improved the explanation of the different ozone‑altering properties of the Antarctic and Arctic;  


(h)
The Antarctic ozone "holes" of 1992 and 1993 were the most severe on record.
  The Antarctic ozone "hole" has continued to occur seasonally every year since it was first observed in the late 1970s, with the occurrences over the last several years being particularly pronounced.  Satellite, balloon‑borne, and ground‑based monitoring instruments revealed that the Antarctic ozone "holes" of 1992 and 1993 were the biggest (areal extent) and deepest (minimum amounts of ozone overhead), with ozone being locally depleted by more than 99% at altitudes between about 14‑19 km in October 1992 and 1993.  It is likely that these larger‑than‑usual ozone depletions could be attributed, at least in part, to sulphate aerosols from Mt. Pinatubo increasing the effectiveness of chlorine‑ and bromine‑catalysed ozone destruction.  A substantial Antarctic ozone "hole" is expected to occur each austral spring for many more decades because stratospheric chlorine and bromine abundances will approach the pre‑Antarctic‑ozone‑"hole" levels (late‑1970s) very slowly during the next century;


(i)
Ozone losses have been detected in the Arctic winter stratosphere, and their links to halogen chemistry have been established.  Studies in the Arctic lower stratosphere have been expanded to include more widespread observations of ozone and key reactive species.  In the late‑winter/early‑spring period, additional chemical losses of ozone up to 15‑20% at some altitudes are deduced from these observations, particularly in the winters of 1991/92 and 1992/93.  Model calculations constrained by the observations are also consistent with these losses, increasing the confidence in the role of chlorine and bromine in ozone destruction.  The interannual variability in the photochemical and dynamical conditions of the Arctic polar vortex continues to limit the ability to predict ozone changes in future years;  


(j)
Methyl bromide continues to be viewed as a significant ozone‑depleting compound.  Increased attention has been focused upon the ozone‑depleting role of methyl bromide.  Three major anthropogenic sources of atmospheric methyl bromide have been identified:  agricultural usage (soil and commodity fumigation), biomass burning, and potentially the exhaust of automobiles using leaded gasoline, in addition to the natural oceanic source.  Recent laboratory studies have confirmed the fast rate for the BrO + HO2 reaction and established a negligible reaction pathway producing HBr, both of which imply greater ozone losses due to emissions of compounds containing bromine.  While the magnitude of the atmospheric photochemical removal is well understood, there are significant uncertainties in quantifying the oceanic sink for atmospheric methyl bromide.  The best estimate for the overall lifetime of atmospheric methyl bromide is 1.3 years, with a range of 0.8‑1.7 years.  The ozone depleting potential (ODP) for methyl bromide is calculated to be about 0.6 (relative to an ODP of 1 for CFC‑11);


(k)
The atmospheric residence times of the important ozone‑depleting gases, CFC‑11 and methyl chloroform, are now better known.  A reconciliation of observed concentrations with known emissions using an atmospheric model has led to a best‑estimate lifetime of 50 years for CFC‑11 and 5.4 years for methyl chloroform, with uncertainties of about 10%.  These lifetimes provide an accurate standard for gases destroyed only in the stratosphere (such as CFCs and nitrous oxide) and for those also reacting with tropospheric hydroxyl radical OH (such as HCFCs and HFCs), respectively;


(l)
The ODP values for new substances have been determined.  The ODPs for several new compounds, such as HCFC-225ca, HCFC-225cb, and iodotrifluoromethane (CF3I), have been evaluated and found to be 0.03 or less;


(m)
Many of the substitutes for the CFCs and halons are also notable greenhouse gases.  Several CFCs and halon substitutes are not addressed under the Montreal Protocol (because they do not deplete ozone), but, because they are greenhouse gases, fall under the purview of the United Nations Framework Convention on Climate Change.  There is a wide range of values for the global warming potentials (GWPs) of the hydroflurocarbons (HFCs), with many of them having values within an order of magnitude of the ozone‑depleting compounds they replace.  The perfluorinated compounds, some of which are being considered as substitutes, have very large GWPs.  These are examples of compounds whose current atmospheric abundances are relatively small, but are increasing or could increase in the future;


(n)
The Montreal Protocol and its Amendments and Adjustments are reducing the impact of anthropogenic halocarbons on the ozone layer and should eventually eliminate this ozone depletion.  Based on assumed compliance by all Parties with the Montreal Protocol as amended at Copenhagen in 1992, the stratospheric chlorine abundances will continue to grow from their current levels (3.6 parts per billion (ppb) to a peak of about 3.8 (ppb) around the turn of the century.  The future total bromine loading will depend upon choices made regarding future human production and emissions of methyl bromide.  After around the turn of the century, the levels of stratospheric chlorine and bromine will decrease during the twenty-first and twenty-second centuries.  The rate of decline is dictated by the long residence times of the CFCs, carbon tetrachloride, and halons.  Global ozone losses and the Antarctic ozone "hole" were first discernible in the late 1970s and recovery is predicted by about 2045, other things being equal.  The recovery of the ozone layer would have been impossible without the Amendments and Adjustments to the original Protocol;


(o)
Peak global ozone losses are expected to occur during the next several years.  The ozone layer will be most affected by human‑influenced perturbations and susceptible to natural variations in the period around 1998, since the peak stratospheric chlorine and bromine abundances are expected to occur then.  Based on extrapolation of current trends, observations suggest that the maximum ozone loss, relative to the late 1960s, will likely be:



  (i)
About 12‑13% at northern mid-latitudes in winter/spring (i.e., about 2.5% above current levels);



 (ii)
About 6‑7% at northern mid-latitudes in summer/autumn (i.e., about 1.5% above current levels); and



(iii)
About 11% (with less certainty) at southern mid-latitudes on a year‑round basis (i.e., about 2.5% above current levels);


(p)
The link between a decrease in stratospheric ozone and an increase in surface ultraviolet (UV) radiation has been further strengthened.  Measurements of UV radiation at the surface under clear‑sky conditions show that low overhead ozone yields high UV radiation and in the amount predicted by radiative‑transfer theory.  Large increases of surface UV are observed in Antarctica and the southern part of South America during the period of the seasonal ozone "hole."  Measurements at Palmer-Station (64 degrees South) in spring time show that the biological effective UV-B radiation when related to DNA-damage can exceed summer values at San Diego, California, United States of America (32 degrees North).  Furthermore, elevated surface UV levels at mid‑to‑high latitudes were observed in the northern hemisphere in 1992 and 1993, corresponding to the low ozone levels of those years.  However, the lack of a decadal (or longer) record of accurate monitoring of surface UV levels and the variation introduced by clouds and other factors have precluded the unequivocal identification of a long‑term trend in surface UV radiation;


(q)
Peak UV radiation increases are expected to occur during the next several years.  The peak global ozone losses would be accompanied by 15% (northern mid-latitudes in winter/spring), 8% (northern mid-latitudes in summer/fall), and 13% (southern mid-latitudes on a year-round basis) increases in surface erythemal radiation, if other influences such as clouds remain constant.  Moreover, if there were to be a major volcanic eruption like that of Mt. Pinatubo, or if an extremely cold and persistent Arctic winter were to occur, then the ozone losses and UV increases could be larger in individual years.


B.  Environmental effects findings
5.
The increased penetration of solar UV-B radiation (290-315 nm) to the Earth's surface has adverse effects on human health, animals, plants, microorganisms, materials and on air quality. 

6.
Increases in UV-B radiation are likely to have substantial adverse effects on human health, including increases in the incidence of and morbidity from skin cancer, eye diseases, and infectious diseases.  In susceptible (light-skinned) populations, UV-B radiation is the key risk factor for the development of non-melanoma skin cancer.  Using information derived from animal experiments and human epidemiology, it is estimated that a sustained 1% decrease in stratospheric ozone will result in an increase of approximately 2% in the incidence of non-melanoma skin cancer.  Epidemiological data indicate that the risk of melanoma increases with sunlight exposure, especially during childhood.  Chronic exposure to UV-B (resulting in a high, cumulative, lifetime dose) is one of several factors clearly associated with the risk of cataracts.  Studies of humans indicate that UV-B radiation can induce suppression of immune systems.  The importance of these immune effects for infectious diseases in humans is unknown.  However, in areas of the world where infectious diseases already pose a significant challenge to human health and in persons with impaired immune function, the added impact of UV-B-induced immune suppression could be significant.

7.
Physiological and developmental processes of plants are adversely affected by UV-B radiation.  Plant growth can be directly reduced by UV-B radiation.  However, plants also have several mechanisms to ameliorate or repair these effects and may acclimatize to a certain extent to increased levels of UV-B.  Secondary effects caused by UV-B (such as changes in plant form, biomass allocation to parts of the plant, timing of developmental phases and secondary metabolism) may be equally, or sometimes more, important than the direct damaging effects of UV-B.  These changes can have important implications for plant competitive balance, herbivory, plant pathogens, and biogeochemical cycles.  These ecosystem-level effects can be anticipated, but not easily predicted or evaluated.  Research at the ecosystem level of solar UV-B impacts is barely beginning.  Other stress factors such as increasing C02 and temperature need to be taken into account for future ecological assessments.

8.
UV-B radiation adversely effects phytoplankton productivity and the early development stages of fish and other aquatic organisms.  Phytoplankton productivity is limited to the euphotic zone, the upper layer of the water column in which phytoplankton organisms can move for sufficient sunlight to support net productivity.  Exposure to solar UV-B radiation has been shown to affect both orientation mechanisms and motility in phytoplankton, resulting in reduced survival rates for these organisms.  Natural phytoplankton communities in the marginal Antarctic ice zone show a direct reduction in phytoplankton production due to ozone-related increases in UV-B.  One study has indicated a 6-12% reduction in phytoplankton production in the marginal ice zone during the period of peak springtime Antarctic ozone depletion.

9.
The most severe effects of solar UV-B radiation are on early developmental stages of fish, shrimp, crab, amphibians and other animals, decreased reproductive capacity, and impaired larval development.  This in turn leads to a significant reduction in the size of the population of consumer organisms.  This is of particular importance because more than 30% of the world's animal protein for human consumption comes from the sea and in many countries, particularly the developing countries, this percentage is significantly higher.  As a result, it is important to know how increased levels of exposure to solar UV-B radiation might affect the productivity of aquatic systems.

10.
Increases in solar UV radiation could decrease the productivity of marine and terrestrial ecosystems, hence reducing the uptake of atmospheric carbon dioxide, and in addition can alter both sources and sinks of greenhouse and chemically-important trace gases, such as carbon monoxide (CO), carbonyl sulphide (COS), and possibly other gases.  Marine phytoplankton are a major sink for atmospheric carbon dioxide, and they have a decisive role in the development of future trends of carbon dioxide concentrations in the atmosphere.  These potential changes would contribute to biosphere-atmosphere feedbacks that attenuate or reinforce the atmospheric build-up of these gases.

11.
New research on the environmental fate and impact of the HFC and HCFC substitutes for CFCs has focused on trifluoroacetate (TFA), a tropospheric oxidation product of certain HFCs and HCFCs.  Although TFA is mildly toxic to some species of marine and freshwater phytoplankton, it will never reach toxic concentrations in oceans or soils because it is biologically degraded. 

12.
Synthetic polymers, naturally occurring biopolymers, as well as some other materials of commercial interest are further adversely affected by increases in solar UV radiation.  The nature and the extent of such damage due to increased UV-B radiation in sunlight is quantified in action spectra showing that the shorter UV-B wavelengths processes are mainly responsible for photo-damage ranging from discoloration to loss of mechanical integrity.

13.
An increase in UV-B radiation (e.g. from stratospheric ozone loss) is expected to decrease tropospheric ozone in the background atmosphere, but, in some cases, it will increase production of ozone in the more-polluted regions.  Increases of UV-B radiation result in higher photodissociation rates of key trace gases that control the chemical reactivity of the troposphere, increasing both production and destruction of ozone and related oxidants such as hydrogen peroxide (H2O2), which are known to have adverse effects on human health, terrestrial plants, and outdoor materials.  Changes in the atmospheric concentrations of the hydroxyl radical (OH) may change the atmospheric lifetimes of greenhouse gases such as methane, HFCs, and HCFCs.


C.  Technology and economics findings

14.
Developed countries, except for a number of countries with economies in transition (CEITs), are generally on schedule to phase out chlorofluorocarbons (CFCs), 1,1,1-trichloroethane, and carbon tetrachloride by 1 January 1996.  The phase-out is virtually complete in most developed countries for manufacture of automobile air conditioners, refrigeration, foams (except limited use for rigid polyurethane foams for home appliance insulation), electronics and aerospace products, and non-medical aerosol products.  For CFCs, individual countries with aggressive early phase-out goals are best prepared for their approaching deadlines.  The European Union (EU) countries are approaching their internal 1 January 1995 CFC phase-out without alarm.  Since the 1995 EU phase-out relies heavily on stockpiling by users, experts wonder whether the transition will be smooth after ozone-depleting substance stockpiles are depleted late in 1995/1996.  The most difficult remaining challenges are for refrigeration and air-conditioning servicing, 1,1,1-trichloroethane solvent use among small and medium-sized industry, metered dose inhalers, precision cleaning of sophisticated aerospace equipment (rocket motors, oxygen systems, and deep-space guidance systems), and for laboratory and analytical applications.  Many laboratory tests are required by government agencies which currently rely on ODSs.  Altering those test requirements will therefore require regulatory change.

15.
HCFCs remain critical for meeting the near-term CFC phase-out goals.  However, they are less important for new equipment produced in the mid- and long-term period.  Available data indicate that it is technically and economically feasible to reduce the HCFC cap due to the rapid development of alternatives and substitutes to HCFCs.  HCFCs are currently necessary for certain new refrigeration and air-conditioning applications, for servicing already installed HCFC equipment, for some rigid thermal insulating and automotive safety foam products, and for several important small uses such as sterilization and precision cleaning, where some existing equipment cannot be converted to non-ODS alternatives.  It is not yet determined whether HCFCs will be required to replace halon in critical uses.  If Article 5 countries are adequately funded and technically assisted, HCFC use could be limited and would eventually decline in these countries as well.

16.
The halon phase-out took effect on 1 January 1994 with little disruption because the fire protection industry had established global information networks and coordinated halon banks.  Halons are in surplus in some countries while in short supply in others.  Halon-1301 banking is fully functional in many countries and being organized in others.  There is increasing interest in the conservation and banking of halon-1211.  Halon banks are important because environmentally acceptable alternatives have not been commercialized for some important fire-protection applications.

17.
Uses of methyl bromide are decreasing in some countries.  Implementation of methyl bromide alternatives and substitutes is virtually complete in the Netherlands with the exception of some quarantine uses, and efforts are at an advanced stage in countries such as Denmark and Italy that have stringent and immediate domestic controls.  Because alternatives to methyl bromide are technically the same in all countries, phase-out schedules for Article 5 countries could be similar to non-Article 5 countries, provided that adequate financing and training is available.  Implementation efforts are intensifying in countries like the United States, Canada, and the members of the European Union that have controls proposed or scheduled for future years. However, despite the clear findings of the science assessment, persistent criticism of the science of ozone depletion by advocates of continued methyl bromide use discourages investment in alternatives.

18.
A new, and apparently increasing, challenge is the smuggling of newly produced CFCs and halon.  Often smugglers attempt to evade prohibition by describing the substances as recycled or as non-CFC chemicals.  These shipments could be primarily from Eastern Europe and developing countries where production facilities are still operating.  Measures that could reduce this illegal activity could be designed in a way to ensure that quantities and procedures are legitimate, rather than to prohibit trade.  This approach would not impede continued recycling which often depends on the ability to ship substances to recycling facilities or to markets where recycling is economic.

19.
Many developing countries are making progress in the phase-out of ODSs in a variety of application areas, but many challenges remain.  ODS solvent use is rapidly decreasing in cases where cooperation exists with multi-national companies and organizations and with strong market incentives.  Where private or Multilateral Fund financing has been available progress appears to be under way in refrigeration and in reducing ODS uses in the manufacture of flexible foam, electronics, and aerosol products.  The banking of recycled halons in both the developed and developing countries is growing sufficiently to provide maintenance quantities for existing equipment.  Developing countries have also made substantial progress in preparing the way for accelerating ODS phase-out by institutional strengthening such as establishing national ozone units, information exchange, preparation of country programmes, and cooperation with the implementing agencies of the Multilateral Fund over investment project proposals.  However, some developing countries have actually increased their use of ODSs due to increased economic growth and constraints in adopting alternatives. 

20.
The points of concern to developing countries include:  availability of ozone-depleting substances (ODSs) to supply developing country basic domestic needs at affordable prices;  apprehension that the phase-out in developed countries may require special efforts to sustain commitment to assist developing countries;  the adequacy of the political and financial support for the Multilateral Fund;  the importance of coordination among the implementing agencies of the Multilateral Fund;  the capacity of developing countries to adopt new technologies;  the barriers to information exchange and;  the reluctance of Governments to promote phase-out activities in view of the

10-year grace period to implement the control measures.

21.
It seems inevitable that compliance of several countries with economies in transition (CEITs) will not occur in 1996 and that significant efforts will be necessary for eventual compliance.  Since the signing of the Montreal Protocol and its London Amendment substantial changes have occurred in the countries of the Central and Eastern European region and the territory of the former Soviet Union.  Some of the States have ratified the Protocol but 10 of the new States are yet to do so.  All States of the Central and Eastern European region as well as Commonwealth of Independent States (CIS) are struggling with the difficulties of the transition from a planned to a market economy, the worldwide recession, market discrimination and exclusion, high inflation rates, and a chronic lack of capital even for investments with short payback time.  The reduction of the consumption of ODSs in the Central and Eastern European States has been a consequence of decrease in industrial output due to the factors referred to earlier, and only partly as a result of actual phase-out activities and investments.  In a number of the successor States of the Soviet Union, there is no legislative basis for either ratifying or implementing the Montreal Protocol and there are no institutions and authorities to manage the phase-out of ODSs or even to report consumption data and to establish the baseline consumption level.  In a number of CEITs, accession to the Protocol as amended in London is difficult because these countries are classified as non-Article 5, a status which includes the obligation to contribute to the Multilateral Fund.  


II.  FEASIBILITY OF OPTIONS FOR LOWERING STRATOSPHERIC CHLORINE


AND BROMINE ABUNDANCES
22.
There are only a limited number of approaches to lowering stratospheric chlorine and bromine abundances beyond those already adopted by the Parties.  Further controls on ozone‑depleting substances would not be expected to significantly change the timing or the magnitude of the peak stratospheric halocarbon abundances and hence peak ozone loss.  However, there are four approaches identified by the Ozone Science Assessment Panel that, if technically and economically feasible, would steepen the initial fall from the peak halocarbon levels in the early decades of the next century:  elimination of methyl bromide from agricultural, structural, and industrial activities; reduce the HCFC cap and accelerate the phaseout schedule;  recover and destroy halons;  and recover and destroy CFCs.

23.
Prior to discussing these options, it should be noted that failure to adhere to the international agreements will delay recovery of the ozone layer.  If there were to be additional production of CFCs at 20% of 1992 levels for each year through 2002 and ramped to zero by 2005 (beyond that allowed for countries operating under Article 5 of the Montreal Protocol), then the integrated effective future chlorine loading above the 1980 level is predicted to be 9% more over the next 50 years relative to full compliance to the Amendments and Adjustments to the Protocol.


A.  Approaches to lowering stratospheric chlorine and bromine abundance 


that are technically and economically feasible


1.  Further control of methyl bromide

24.
The Ozone Science Assessment Panel calculated that if emissions of methyl bromide from agricultural, structural, and industrial activities were to be eliminated in the year 2001, then the integrated effective future chlorine loading above the 1980 level (which is related to the cumulative future loss of ozone) is predicted to be 13% less over the next 50 years relative to full compliance to the Amendments and Adjustments to the Protocol.

25.
The controls under the current Protocol freeze methyl bromide consumption by January 1995 at 1991 levels in non-Article 5 countries.  It is the consensus of the Technology and Economic Assessment Panel (TEAP) and its Methyl Bromide Technical Options Committee that further controls beyond the currently established control levels are technically feasible.  The TEAP believes that additional controls on methyl bromide would be more cost-effective per ODP-weighted tonne than additional controls on other controlled substances.  Progress beyond a consumption freeze appears possible since several developed nations have already achieved or imposed stricter controls on methyl bromide use.  For example, between 1981 and 1991 the Netherlands eliminated its use of methyl bromide in soil fumigation through the use of chemicals and non-chemical alternatives such as improved steam sterilization techniques, artificial and natural growth substrates, resistant plant species, crop rotation, and chemical substitutes.  Reductions could be accomplished through increased training;  prompt registration of chemical substitutes and not-in-kind alternatives;  and substantial investment in containment, recovery equipment, food storage facilities, and other equipment.  Comparable investments have already been made in other application areas in order to protect the ozone layer.  Using known technology it is technically possible to reduce usage of methyl bromide.  It is not technically or economically feasible to phase-out all uses.  Estimates of the magnitude of the reduction and its time-scale varied widely among Methyl Bromide Technical Options Committee members.  Opinions ranged from a reduction of 50% feasible by 1998, to decreases of only a few percent by 2001.  While alternatives are available for the majority of current uses, technically feasible alternatives are not available for less than 10% of 1991 methyl bromide uses.  Many of these applications without currently identified alternatives and substitutes are for treatment of exports to meet quarantine standards, where extensive trials and protracted bilateral negotiations may be required.  Of course, these uses are currently exempted under the Protocol.


2.  Reductions in the HCFC cap and acceleration of the phase-out schedule

26.
The Ozone Science Assessment Panel calculated that if emissions of HCFCs were to be totally eliminated by the year 2004, then the integrated effective future chlorine loading above the 1980 level is predicted to be 5% less over the next 50 years relative to full compliance with the Amendments and Adjustments to the Protocol.

27.
As noted earlier, HCFCs remain critical for meeting the near-term CFC phase-out goals.  They are less important for new equipment available in the mid and long-term period.  Few large scale current uses of HCFCs have been reported for solvents, coatings, or adhesives.  Available data indicate that it is technically feasible to further limit HCFC consumption by encouraging selection of non-HCFC substitutes to replace CFC, halon, and 1,1,1-trichloroethane;  by better containing HCFCs;  and by increased recycling.  However, a complete phase-out by 2004 is not technically feasible with currently available technology.

28.
In considering further HCFC controls, the Parties may wish to take into account that:


(a)
Many non-HCFC alternatives to CFCs, halons, 1,1,1-trichloroethane and carbon tetrachloride present their own environmental and worker risks.  If HCFCs are not available, users may select more toxic chemicals, potent greenhouse gases such as perfluorinated compounds (PFCs), or refrigerants that are not energy efficient or do not have demonstrated long-term reliability.  Forcing the early switch from HCFCs to these options could discourage later selection of more environmentally acceptable alternatives because of the prohibitive cost of investment; 


(b)
HCFCs are sometimes "stepping stones" to next-generation technologies that would either not be possible or would be far more difficult to implement without the technical lessons of HCFC use; and


(c)
Some companies demonstrating environmental leadership--at the encouragement of the Protocol, national Governments, and non-government organizations--selected HCFC technologies long before they were proven or practical in an effort to shift away from CFCs at the earliest possible time.  This leadership helped create global markets for technologies that protect the zone layer.  Parties may wish to consider the advantages of not "turning leaders into losers" by halting HCFC production too soon to allow reasonable recovery of the investment costs.  This is particularly true where equipment has a long useful economic life.


B.  Approaches to lowering stratospheric chlorine and bromine abundances 


that are not technically and/or economically feasible


1.  Recover and destroy halon

29.
The Ozone Science Assessment Panel calculated that if halons presently contained in existing equipment were never released to the atmosphere, the integrated effective future chlorine loading above the 1980 level is predicted to be 10% less over the next 50 years relative to full compliance with the Amendments and Adjustments to the Protocol.

30.
Although it is technically feasible to destroy halon, existing halon stocks are required at the present time for critical uses, representing 15-20% of current uses, which now have no identified substitutes or alternatives.  Thus, a substantial portion of already produced halon may be necessary to supply essential uses where human life and irreplaceable property is protected. 

31.
The decision at the Fourth Meeting of the Parties to bring forward the phase-out of halons to 1994 was predicated on the prudent use of banked halons.  The halon phase-out took effect 1 January 1994 with little disruption because the fire protection industry had established global information networks and coordinated halon banks.  Halons are in surplus in some countries while in short supply in others.  Halon 1301 banking is fully functional in many countries and being organized in others.  There is increasing conservation and banking of halon 1211.

32.
Recovery of halon from non-critical existing uses is proceeding in most countries as a result of the proactive efforts of the fire-protection community.  Parties could further promote the recovery of halon in non-critical uses for essential uses in non-Article 5 and Article 5 countries.  Moreover, the promotion of mandatory destruction by owners is likely to be counter-productive since it would discourage halon banking and lead to the discharging of halon into the atmosphere.    


2.  Recover and destroy CFCs
33.
The Ozone Science Assessment Panel calculated that if CFCs presently contained in existing equipment were never released to the atmosphere, the integrated effective future chlorine loading above the 1980 level is predicted to be 3% less over the next 50 years relative to full compliance with the Amendments and Adjustments to the Protocol.  

34.
It is presently technically but not economically feasible to destroy CFCs  because recycled CFCs are necessary to service existing equipment.  Most non-Article 5 Parties, and particularly the European Union (EU) with a CFC phase-out in January 1995, may require substantial quantities of CFCs from recovery in order to service existing equipment during the transition to substitute refrigerants.  Recovery and destruction of needed CFCs from refrigeration and air-conditioning equipment would increase the dependence on HCFCs and HCFC blends and would be very costly if equipment were prematurely abandoned.  Expensive recycling equipment and infrastructure have been developed to protect the ozone layer.  It is also likely that CFC shortages would intensify smuggling of CFCs and dumping of CFC-dependent equipment in developing countries. 


III.  FUTURE ASSESSMENT WORK:  HCFC AND METHYL BROMIDE ELABORATION


(DECISION VI/13), CHALLENGES FACING ARTICLE 5 COUNTRIES

 
AND COUNTRIES WITH ECONOMIES IN TRANSITION

 
AND ESSENTIAL USES

35.
The Sixth Meeting of the Parties requested that the Assessment Panels further elaborate by March 1995 the feasibility and implications of alternatives and substitutes to HCFCs and methyl bromide.  The March 1995 report will include an evaluation of the feasibility of reducing the HCFC cap, accelerating the initial reductions, and/or speeding the phase-out.  This assessment will also describe the history of technical development in applications where HCFCs were the first feasible alternatives to CFCs but may no longer be required.  A full range of methyl bromide control options will be considered. 

36.
The March 1995 report will also include updated information on the challenges faced by Article 5 countries and countries with economies in transition.

37.
In 1994, the essential use nominations process was successful and collaborative.  In many cases, the TOC experts identified suitable alternatives or substitutes or helped guide applicants through rapid identification and development.  The 1995 report will include evaluations of new nominations for exemption to the phase-out of production for essential uses and will update progress on uses that were granted exemptions in 1994.


Appendix


LIST OF SCIENCE, ENVIRONMENTAL EFFECTS, AND


TECHNOLOGY AND ECONOMIC ASSESSMENT EXPERTS


Science Assessment Panel 

Co-chairs

Daniel L. Albritton, Robert T. Watson, and Piet J. Aucamp

Chapter Lead Authors

1.   Neil R.P. Harris

2.   Eugenio Sanhueza

3.   David W. Fahey

4.   Roderic L. Jones

5.   Andreas Volz‑Thomas and Brian A. Ridley

6.   Malcolm K.W. Ko 

7.   Frode Stordal

8.   Keith P. Shine 

9.   Richard L. McKenzie 

10.  Stuart A. Penkett

11.  Andreas Wahner and Marvin A. Geller

12.  R. Anthony Cox 

13.  Susan Solomon and Donald J. Wuebbles

Name
Affiliation
Country     

Daniel L. Albritton
NOAA Aeronomy Laboratory
United States of America

Marc Allaart
Koninklijk Nederlands Meteorologisch Instituut
Netherlands

Fred N. Alyea
Georgia Institute of Technology
United States of America

Gerard Ancellet
Centre National de la Recherche Scientifique
France

Meinrat O. Andreae
Max-Planck-Institut für Chemie
Germany

James K. Angell
NOAA Air Resources Laboratory
United States of America

Frank Arnold
Max-Planck-Institut für Kernphysik
Germany

Roger Atkinson
University of California at Riverside
United States of America

Elliot Atlas
National Center for Atmospheric Research
United States of America

Piet J. Aucamp
Department of Health
South Africa

Linnea Avallone
University of California at Irvine
United States of America

Helmuth Bauer
Forschungszentrum für Umwelt u. Gesundheit
Germany

Slimane Bekki
University of Cambridge
United Kingdom

Tibor Bérces
Hungarian Academy of Sciences
Hungary

T. Berntsen
Universitetet I Oslo
Norway

Lane Bishop
Allied Signal      
United States of America

Donald R. Blake
University of California at Irvine
United States of America

N.J. Blake
University of California at Irvine
United States of America

Mario Blumthaler
University of Innsbruck
Austria

Greg E. Bodeker
University of Natal/NIWA
South Africa

Rumen D. Bojkov
World Meteorological Organization
Switzerland

Charles R. Booth
Biospherical Instruments
United States of America

Byron Boville
National Center for Atmospheric Research
United States of America

Kenneth P. Bowman
Texas A&M University
United States of America

Geir Braathen
Nosk Institutt for Luftforskning
Norway

Guy P. Brasseur
National Center for Atmospheric Research
United States of America

Carl Brenninkmeijer
National Institute of Water and Atmospheric


 Research 
New Zealand

William H. Brune
Max-Planck-Institut für Chemie
Germany

Christoph Brühl
Pennsylvania State University
United States of America

James H. Butler
NOAA Climate Monitoring and
United States of America


 Diagnostics Laboratory

Sergio Cabrera
Universidad de Chile
Chile

Bruce A. Callander
United Kingdom Meteorological Office
United Kingdom

Daniel Cariolle
Météo-France Centre National de Recherches


  Météorologiques
France

Richard P. Cebula
Hughes STX
United States of America

William L. Chameides
Georgia Institute of Technology
United States of America

Sushil Chandra
NASA Goddard Space Flight Center
United States of America

Marie‑Lise Chanin
Centre National de la Recherche Scientifique
France

J. Christy
University of Alabama at Huntsville
United States of America

Ralph J. Cicerone
University of California at Irvine
United States of America

G.J.R. Coetzee
Weather Bureau
South Africa

Peter S. Connell
Lawrence Livermore National Laboratory
United States of America

D. Considine
NASA Goddard Space Flight Center
United States of America

R. Anthony Cox
National Environmental Research Council
United Kingdom

Paul J. Crutzen
Max-Planck-Institut für Chemie
Germany

Derek N. Cunnold
Georgia Institute of Technology
United States of America

John Daniel
NOAA Aeronomy Laboratory/CIRES
United States of America

Malgorzata Degórska
Polish Academy of Sciences
Poland

John J. DeLuisi
NOAA Air Resources Laboratory              
United States of America

Dirk De Muer
Institut Royal d'Météorologigue de Belgique
Belgium

Frank Dentener
Wageningen Agricultural University
Netherlands

Richard G. Derwent
UK Meteorological Office
United Kingdom

Terry Deshler
University of Wyoming
United States of America

Susana B. Diaz
Austral Center of Scientific Research


  (CADIC/CONICET)
Argentina

Russell Dickerson
University of Maryland
United States of America

J. Dignon
Lawrence Livermore National Laboratory
United States of America

Ed Dlugokencky
NOAA Climate Monitoring and Diagnostics Lab
United States of America

Anne R. Douglass
NASA Goddard Space Flight Center
United States of America

Tom Duafala
Methyl Bromide Global Coalition
United States of America

James E. Dye
National Center for Atmospheric Research
United States of America

Dieter H. Ehhalt
Forschungszentrum Jülich   
Germany

James W. Elkins
NOAA Climate Monitoring and Diagnostics Lab
United States of America

Christine Ennis
NOAA Aeronomy Laboratory/CIRES
United States of America

D. Etheridge
CSIRO Division of Atmospheric Research
Australia

David W. Fahey
NOAA Aeronomy Laboratory
United States of America

T. Duncan A. Fairlie
NASA Langley Research Center
United States of America

Donald A. Fisher
E.I. DuPont de Nemours and Company
United States of America

Jack Fishman
NASA Langley Research Center
United States of America

Eric L. Fleming
Applied Research Corporation
United States of America

Frank Flocke
Forschungszentrum Jülich
Germany

Lawrence E. Flynn
Software Corporation of America
United States of America

P.M. de F. Forster
University of Reading
United Kingdom

James Franklin
Solvay S.A.
Belgium

Paul J. Fraser
CSIRO Division of Atmospheric Research
Australia

John E. Frederick
University of Chicago
United States of America

Lucien Froidevaux
California Institute of Technology/Jet Propulsion


 Laboratory
United States of America

J.S. Fuglestvedt
Center for International


 Climate & Energy Research
Norway

Reinhard Furrer
Freie Universitat Berlin
Germany

Ian E. Galbally
CSIRO Division of Atmospheric Research
Australia

Brian G. Gardiner
British Antarctic Survey
United Kingdom

Marvin A. Geller
State University of New York at Stony Brook
United States of America

Hartwig Gernandt
Alfred Wegener Institut
Germany

James F. Gleason
NASA Goddard Space Flight Center
United States of America

Sophie Godin
Centre National de la Recherche Scientifique
France

Amram Golombek
Israel Institute for Biological Research
Israel

Ulrich Görsdorf
Deutscher Wetterdienst
Germany

Thomas E. Graedel
AT&T Bell Laboratories
United States of America

Claire Granier
National Center for Atmospheric Research
United States of America

William B. Grant
NASA Langley Research Center
United States of America

L.J. Gray
SERC Rutherford Appleton Lab
United Kingdom

William A. Grose
NASA Langley Research Center
United States of America

J. Gross
Max-Planck-Institut für Chemie
Germany

A.S. Grossman
Lawrence Livermore National Laboratory
United States of America

Alexander Gruzdev
Russian Academy of Sciences
Russian Federation

James E. Hansen
NASA Goddard Institute for Space Studies
United States of America

Neil R.P. Harris
European Ozone Research Coordinating Unit
United Kingdom

Shiro Hatekeyama
National Institute for the Environment
Japan

D.A. Hauglustaine
Centre National de la Recherche Scientifique
France

Sachiko Hayashida
Nara Women's University
Japan

G.D. Hayman
Harwell Laboratory/AEA Environment and


 Energy
United Kingdom

Kjell Henriksen
University of Tromsø
Norway

Ernest Hilsenrath
NASA Goddard Space Flight Center
United States of America

David J. Hofmann
NOAA Climate Monitoring and Diagnostics Lab
United States of America

Stacey M. Hollandsworth
Applied Research Corporation
United States of America

James R. Holton
University of Washington
United States of America

Lon L. Hood
University of Arizona
United States of America

Øystein Hov
University of Bergen
Norway

Carleton J. Howard
NOAA Aeronomy Laboratory
United States of America

Robert D. Hudson
University of Maryland
United States of America

Drusilla Hufford
Environmental Protection Agency
United States of America

Linda Hunt
NASA Langley Research Center
United States of America

Abdel M. Ibrahim
Egyptian Meteorological Authority
Egypt

Mohammad Ilyas
University of Science Malaysia
Malaysia

Ivar S. A. Isaksen
Universitetet I Oslo
Norway

Tomoyuki Ito
Japan Meteorological Agency
Japan

Charles H. Jackman
NASA Goddard Space Flight Center
United States of America

Daniel J. Jacob
Harvard University
United States of America

Colin E. Johnson
UK Meteorological Office/AEA Technology
United Kingdom

Harold S. Johnston
University of California at Berkeley
United States of America

Paul V. Johnston
National Institute of Water & Atmospheric


 Research
New Zealand

Roderic L. Jones
University of Cambridge
United Kingdom

Torben S. Jørgensen
Danish Meteorological Institute
Denmark

Maria Kanakidou
Centre National de la Recherche Scientifique
France

Igor L. Karol
A.I. Voeikov Main Geophysical Observatory
Russian Federation

Prasad Kasibhatla
Georgia Institute of Technology
United States of America

Jack A. Kaye
NASA Goddard Space Flight Center
United States of America

Hennie KelderKoninklijk 
Nederlands Meteorologisch Instituut
Netherlands

James B. Kerr
Atmospheric Environment Service
Canada

M.A.K. Khalil
Oregon Graduate Institute of Science and


  Technology       
United States of America

Vyacheslav Khattatov
Central Aerological Observatory
Russian Federation

Jeffrey T. Kiehl
National Center for Atmospheric Research
United States of America

Stefan Kinne
NASA Ames Research Center
Germany

D. Kinnison
Lawrence Livermore National Laboratory
United States of America

Volker Kirchhoff
Instituto Nacional de Pesquisas Espaciais
Brazil

Malcolm K.W. Ko
Atmospheric and Environmental Research      
United States of America

Ulf Köhler
Deutscher Wetterdienst
Germany

Walter D. Komhyr
NOAA Climate Monitoring and Diagnostics Lab
United States of America

Yutaka Kondo
Nagoya University
Japan

Janusz W. Krzy_cin
Polish Academy of Sciences
Poland

Antti Kulmala
World Meteorological Organization
Switzerland

Michael J. Kurylo
NASA Headquarters/NIST
United States of America

Karin Labitzke
Freie Universität Berlin
Germany

Murari Lal
Indian Institute of Technology
India

K.S. Law
University of Cambridge
United Kingdom

G. LeBras
Centre National de la Recherche Scientifique
France

Yuan‑Pern Lee
National Tsing Hua University
Taiwan

Franck Lefèvre
Météo France CNRM
France

Jos Lelieveld
Wageningen University
Netherlands

Robert Lesclaux
Université de Bordeaux 1
France

Joel S. Levine
NASA Langley Research Center
United States of America

Joel Levy
NOAA Office of Global Programs
United States of America

J. Ben Liley
National Insitutute of Water & Atmospheric Research
New Zealand

Peter Liss
University of East Anglia
United Kingdom

David H. Lister
Defence Research Agency
United Kingdom

Zenobia Lity_ska
Centre of Aerology
Poland

Shaw C. Liu
NOAA Aeronomy Laboratory
United States of America

Jennifer A. Logan
Harvard University
United States of America

Nicole Louisnard
Office National d'Etudes et Recherches 


  Aerospatiales
France

Pak Sum Low
United Nations Environment Programme
Kenya

Daniel Lubin
University of California at San Diego
United States of America

Sasha Madronich
National Center for Atmospheric Research
United States of America

Jerry Mahlman
NOAA Geophysical Fluid Dynamics Laboratory
United States of America

Gloria L. Manney
California Institute of Technology/Jet Propulsion


  Laboratory
United States of America

Huiting Mao
State University of New York at Albany/ASRC
United States of America

W. Andrew Matthews
National Institute of Water & Atmospheric Research
New Zealand

Konrad Mauersberger
Max-Planck-Institut für Kernphysik
Germany

Archie McCulloch
ICI Chemicals and Polymers Limited
United Kingdom

Mack McFarland
E.I. DuPont de Nemours and Company
United States of America

Michael E. McIntyre
University of Cambridge
United Kingdom

Richard L. McKenzie
National Institute of Water & Atmospheric Research
New Zealand

Richard D. McPeters
NASA Goddard Space Flight Center
United States of America

Gerard Mégie
Centre National de la Recherche Scientifique
France

Paulette Middleton
Science and Policy Associates
United States of America

Alvin J. Miller
NOAA National Meteorological Center
United States of America

Igor Mokhov
Institute of Atmospheric Physics
Russian Federation

Mario Molina
Massachusetts Institute of Technology
United States of America

Geert K. Moortgat
Max-Planck-Institute für Chemie
Germany

Hideaki Nakane
National Institute for Environmental Studies
Japan

Paul A. Newman
NASA Goddard Space Flight Center
United States of America

Paul C. Novelli
University of Colorado
United States of America

Samuel J. Oltmans
NOAA Climate Monitoring and Diagnostics Lab
United States of America

Alan O'Neill
University of Reading
United Kingdom

Michael Oppenheimer
Environmental Defense Fund
United States of America

S. Palermi
Universita' degli Studi-l'Aquila
Italy

Ken Patten
Lawrence Livermore National Laboratory
United States of America

Juan Carlos Pelaez
Instituto de Meteorologia
Cuba

Stuart A. Penkett
University of East Anglia
United Kingdom

Joyce Penner
Lawrence Livermore National Laboratory
United States of America

Thomas Peter
Max-Planck-Institut für Chemie
Germany

Leon F. Phillips
University of Canterbury
New Zealand

Ken Pickering
NASA Goddard Space Flight Center
United States of America

R. Bradley Pierce
NASA Langley Research Center
United States of America

S. Pinnock
University of Reading
United Kingdom

Michel Pirre
Centre National de la Recherche Scientifique
France

Giovanni Pitari
Universita' degli Studi-l'Aquila
Italy

Walter G. Planet
NOAA National Environmental Satellite, Data and 


  Information Service
United States of America

R. Alan Plumb
Massachusetts Institute of Technology
United States of America

Jean‑Pierre Pommereau
Centre National de la Recherche Scientifique
France

Lamont R. Poole
NASA Langley Research Center
United States of America

Michael J. Prather
University of California at Irvine
United States of America

Margarita Préndez
Universidad de Chile
Chile

Ronald G. Prinn
Massachusetts Institute of Technology
United States of America

Joseph M. Prospero
University of Florida
United States of America

John A. Pyle
University of Cambridge
United Kingdom

Lian Xiong Qiu
Academia Sinica
China

Richard Ramaroson
Office National d'Etudes et de Recherches 


  Aérospatiales   
France

V. Ramaswamy
NOAA Geophysical Fluid Dynamics Laboratory/


  Princeton University
United States of America

William Randel
National Center for Atmospheric Research
United States of America

Phillip J. Rasch
National Center for Atmospheric Research
United States of America

A.R. Ravishankara
NOAA Aeronomy Laboratory
United States of America

William S. Reeburgh
University of California at Irvine
United States of America

C.E. Reeves
University of East Anglia
United Kingdom

J. Richardson
NASA Langley Research Center
United States of America

Brian A. Ridley
National Center for Atmospheric Research
United States of America

David Rind
NASA Goddard Institute for Space Studies
United States of America

Curtis P. Rinsland
NASA Langley Research Center
United States of America

Aidan E. Roche
Lockheed Corporation
United States of America

Michael O. Rodgers
Georgia Institute of Technology
United States of America

Henning Rodhe
Stockholm University
Sweden

Jose M. Rodriguez
Atmospheric and Environmental Research
United States of America

M. Roemer
TNO Institute of Environmental Sciences
Netherlands

Franz Rohrer
Forschungszentrum Jülich
Germany

Richard B. Rood
NASA Goddard Space Flight Center
United States of America

F. Sherwood Rowland
University of California at Irvine
United States of America

Colin E. Roy
Australian Radiation Laboratory
Australia

Jochen Rudolph
Institut für Chemie and Dynamik der Geosphäre
Germany

James M. Russell, III
NASA Langley Research Center
United States of America

Nelson Sabogal
United Nations Environment Programme
Kenya

Karen Sage
NASA Langley Research Center
United States of America

Ross Salawitch
Harvard University
United States of America

Eugenio Sanhueza
Instituto Venezolano de Investigaciones Cientificas
Venezuela

K.M. Sarma
United Nations Environment Programme
Kenya

Toru Sasaki
Meteorological Research Institute
Japan

Susan M. Schauffler
National Center for Atmospheric Research
United States of America

Hans Eckhart Scheel
Fraunhofer Institut für Atmosphärische 


  Umweltforschung
Germany

Ulrich Schmidt
Forschungszentrum Jülich
Germany

Rainer Schmitt
Meteorologie Consult
Germany

Ulrich Schumann
DLR Institut für Physik der Atmosphare
Germany

M.D. Schwarzkopf
NOAA Geophysical Fluid Dynamics Laboratory
United States of America

Gunther Seckmeyer
Fraunhofer Institute for Atmospheric Environment
Germany

Jonathan D. Shanklin
British Antarctic Survey
United Kingdom

Keith P. Shine
University of Reading
United Kingdom

H.W. Sidebottom
University College Dublin
Ireland

P. Simmonds
University of Bristol
United Kingdom

Paul C. Simon
Institut d'Aeronomie Spatiale de Belgique
Belgium

Hanwant B. Singh
NASA Ames Research Center
United States of America

Paula Sk_ivánková
Hydrometeorological Institute, Department of 


  Upper-air  Meteorological Measurement 


  (CHMU/OAP)
Czech Republic

Herman Smit
Forschungszentrum Jülich
Germany

Susan Solomon
NOAA Aeronomy Laboratory
United States of America

Johannes Staehelin
Eidgenossische Technische Hochschule Zürich
Switzerland

Knut Stamnes
University of Alaska
United States of America

L. Paul Steele
CSIRO Division of Atmospheric Research
Australia

Leopoldo Stefanutti
Instituto di Riccrea sulle Onde Elettromagnetiche 


  del CNR
Italy

Richard S. Stolarski
NASA Goddard Space Flight Center
United States of America

Frode Stordal
Norsk Institutt for Luftforskning
Norway

A. Strand
University of Bergen
Norway

B.H. Subbaraya
Physical Research Laboratory
India

Nien‑Dak Sze
Atmospheric and Environmental Research      
United States of America

Anne M. Thompson
NASA Goddard Space Flight Center
United States of America

Xue X. Tie
National Center for Atmospheric Research
United States of America

Margaret A. Tolbert
University of Colorado
United States of America

Darin W. Toohey
University of California at Irvine
United States of America

Ralf Toumi
University of Cambridge
United Kingdom

Michael Trainer
NOAA Aeronomy Laboratory      
United States of America

Charles R. Trepte
NASA Langley Research Center
United States of America

Adrian Tuck
NOAA Aeronomy Laboratory
United States of America

R. van Dorland
Koninklijk Nederlands Meteorologisch Instituut
Netherlands

Karel Vanicek
Czech Hydrometeorological Institute
Czech Republic

Geraint Vaughan
University of Wales
United Kingdom

Guido Visconti
Universita' degli Studi-l'Aquila
Italy

Andreas Volz‑Thomas
Forschungszentrum Jülich
Germany

Andreas Wahner
Forschungszentrum Jülich
Germany

Wei‑Chyung Wang
State University of New York at Albany; ASRC
United States of America

David I. Wardle
Atmospheric Environment Service
Canada

David A. Warrilow
UK Department of the Environment
United Kingdom

Joe W. Waters
California Institute of Technology/Jet Propulsion 


  Laboratory
United States of America

Robert T. Watson
Office of Science and Technology Policy
United States of America

E.C. Weatherhead
NOAA Air Resources Laboratory
United States of America

Christopher R. Webster
California Institute of Technology/Jet Propulsion 


  Laboratory
United States of America

D. Weisenstein
Atmospheric and Environmental Research      
United States of America

Ray F. Weiss
Scripps Institution of Oceanography  
United States of America

Paul Wennberg
Harvard University
United States of America

Howard Wesoky
National Aeronautics and Space Administration
United States of America

Thomas M.L. Wigley
University Corporation for Atmospheric Research
United States of America

Oliver Wild
University of Cambridge
United Kingdom

Paul H. Wine
Georgia Institute of Technology
United States of America

Peter Winkler
Deutscher Wetterdienst
Germany

Steven C. Wofsy
Harvard University
United Stats of America

Donald J. Wuebbles
University of Illinois
United States of America

Vladimir Yushkov
Central Aerological Observatory
Russian Federation

Ahmed Zand
Tehran University
Islamic Republic of Iran

Rudi J. Zander
University of Liege
Belgium

Joseph M. Zawodny
NASA Langley Research Center
United States of America

Reinhard Zellner
Universitat Gesamthochschule Essen
Germany

Christos Zerefos
Aristotle University of Thessaloniki
Greece

Xiu Ji Zhou
Academy of Meteorological Science
China

Environmental Effects Assessment Panel 
Chairs 
Affiliation           
Country     

Xiaoyan Tang
Peking University
China

Manfred Tevini
University of Karlsruhe
Germany

Jan C. van der Leun
University Hospital Utrecht
Netherlands


Environmental Effects Assessment Panel Members
Name
Affiliation
Country
Mohammed B. Amin
King Fahd University of Petroleum and Minerals
Saudi Arabia

Anthony Andrady
Research Triangle Institute
United States of America

Lars Olof Björn
Lund University
Sweden

Janet F. Bornman
Lund University
Sweden

Martyn Caldwell
Utah State University
United States of America

Terry Callaghan
University of Manchester
United Kingdom

Frank R. de Gruijl
University Hospital Utrecht
Netherlands

David Erickson
National Center for Atmospheric Research
United States of America

Donat-P. Häder
University of Erlangen-Nurnberg
Germany

Haleem Hamid
King Fahd University of Petroleum and Minerals
Saudi Arabia

Xingzhou Hu
Academia Sinica Research Institute of Chemistry
China

Margaret L. Kripke
The University of Texas
United States of America

G. Kulandaivelu
Madurai Kamaraj University
India

H. D. Kumar
Banaras Hindu University
India

Janice D. Longstreth
Waste Policy Institute
United States of America

Sasha Madronich
National Center for Atmospheric Research
United States of America

Richard L. McKenzie
National Institute of Water and Atmospheric 


  Research
New Zealand

Raymond C. Smith
University of California at Santa Barbara
United States of America

Yukio Takizawa
Akita University School of Medicine
Japan

Alan H. Teramura
University of Hawaii
United States of America

Ayako Torikai
Nagoya University
Japan

Robert C. Worrest
Information Network (CIESIN)
United States of America

Richard G. Zepp
Environmental Protection Agency
United States of America


Environmental Effects Assessment Expert Reviewers
Name
Affiliation
Country
Meinrat O. Andreae
Max-Planck Institute für Chemie
Germany

Bruce K. Armstrong
Australian Institute of Health and Welfare
Australia

P. J. Aucamp
Department of Health 
South Africa

Carlos L. Ballaré
University of Buenos Aires
Argentina

Safa Baydoun
Atomic Energy Commission
Syria

Michel Boko
Laboratory of Climatology
Benin

S. M. Cayless
Department of the Environment, Global


  Atmosphere Division
United Kingdom

J. P. Césarini
INSERM, Foundation Ophtalmologique
France

Forrest G. Chumley
E.I. DuPont de Nemours and Company
United States of America

Thomas P. Coohill
Ultraviolet Consultants
United States of America

J.E. Costa Martins
University of Sao Paulo
Brazil

Anthony P. Cullen
School of Optometry, University of Waterloo
Canada

Günter Döhler
Botanisches Instiut
Germany

Edward DeFabo
George Washington University Medical Center
United States of America

Michael J. Doughty
School of Optometry, University of Waterloo
Canada

Gun_rs Duburs       
Latvian Institute of Organic Synthesis
Latvia

Alex E. S. Green
University of Florida
United States of America

R. Guicherit
TNO Institute of Environmental Studies
Netherlands

Mohammad Ilyas
University of Science of Malaysia
Malaysia

John Jagger
Consultant
United States of America

H. van Loveren
National Institute for Public Health and 
Netherlands


  Environmental Protection

Ali G. Maadhah
King Fahd University of Petroleum and Minerals
Saudi Arabia

Mack McFarland       
E.I. DuPont de Nemours and Company
United States of America

A. J. McMichael
London School of Hygiene and Tropical Medicine
United Kingdom

Giuliana Moreno
Museum National d'Historie Naturelle
France

Gillian Murphy
Beaumont Hospital
Ireland

David Olszyk
Environmental Research Laboratory
United States of America

Hugh M. Pitcher
Pacific Northwest Laboratory
United States of America

Donald G. Pitts
Consultant
United States of America

Barbara Prézelin
University of California at Santa Barbara
United States of America

Werner Rau
Botanisches Institut der Universite Muenchen
Germany

Henning Rodhe
University of Stockholm
Sweden

Norma D. Searle
Consultant
United States of America

Zden_k Šesták
Academy of Sciences of the Czech Republic
Czech Republic

Richard B. Setlow
Brookhaven National Laboratory
United States of America

Richard Soulen
Technical and Management Services      
United States of America

Hugh R. Taylor
Melbourne University
Australia

F. Urbach
Temple University Medical Practices
United States of America

K. Victorin
Karlinska Institute of Environmental Medicine
Sweden

Alan R. Wellburn
Lancaster University
United Kingdom

Eckard Wellmann
Biologisches Institut II der Universitaet
Germany

Technology and Economic Assessment Panel and Options Committees 

Technology and Economic Assessment Panel (TEAP)
Chairs
Affiliation
Country
Stephen O. Andersen
Environmental Protection Agency
United States of America

Suely Carvalho
University of Sao Paulo
Brazil

Lambert Kuijpers
Technical University Eindhoven
Netherlands

Senior Advisors
Affiliation
Country
Laszlo Dobo
Ministry for Environment and Regional Policy 
Hungary

Yuichi Fujimoto
Japan Electrical Manufacturers' Association
Japan

Carmelina Lombardi
Environment Ministry, Regional Planning
Venezuela

Other Panel Members
Affiliation
Country
Jonathan Banks
Commonwealth Scientific and Industrial Research 
Australia


  Organization (CSIRO)

Andrea Hinwood
Monash University
Australia

Jean Lupinacci
Environmental Protection Agency
United States of America

Tom Morehouse 
Institute for Defense Analyses
United States of America

Jose Pons Pons
Spray Quimica C.A.
Venezuela

Sally Rand 
Environmental Protection Agency
United States of America

Gary Taylor
Taylor/Wagner     
Canada

Helen Tope
Environment Protection Authority, Victoria
Australia

Robert Van Slooten
Department of Trade and Industry
United Kingdom


TEAP Aerosols, Sterilants, Miscellaneous Uses and


Carbon Tetrachloride Technical Options Committee
Chairs
Affiliation
Country
Andrea Hinwood
Monash University
Australia

Jose Pons Pons
Spray Quimica C.A.
Venezuela

Helen Tope
Environmental Protection Authority, Victoria
Australia

Members
Affiliation
Country
A.J. Barnes
Boehringer Ingelheim 
Germany

Nick Campbell
ICI KLEA
United Kingdom

S. W. Clarke
The Royal Free Hospital
United Kingdom

J.R. Claude*
Universit Descartes
France

Francis M. Cuss
Schering-Plough Research
United States of America

Donald Dunn
E.I. DuPont de Nemours and Company
United States of America

Charles Hancock
MDT Corporation
United States of America

Anders Hansson
Astra Draco AB
Sweden

Katsuo Imazeki*
Tokyo Aerosol Industry Company
Japan

Montfort Johnson*
Montfort A. Johnson & Associates
United States of America

R. C. Knollys*
FEA Environment Committee
United Kingdom

Shigeo Kojima*
National Institute of Hygienic Sciences
Japan

P. Kumarasamy
Kontrak Manufacturing Services
Malaysia

Hiroshi Kurita
Japan Association for Hygiene of Chloronated


  Solvents (JAHCS)
Japan

Rob Layet*
Ensign Laboratories
Australia

Robert F. Morrissey*
Johnson & Johnson
United States of America

Geno Nardini
Inst. Internacional del Aerosol
Mexico

Dick Nusbaum
Pennsylvania Engineering Company
United States of America

Martyn Partridge
Whipps Cross Hospital
United Kingdom

Abe Rubinfeld
Royal Melbourne Hospital
Australia

Birgitta Schmekel
University Hospital
Sweden

Albert L. Sheffer
Brigham and Women`s Hospital
United States of America

Greg Simpson
CSIRO
Australia

Ian Smith
Glaxo Group R&D 
United Kingdom

Robert Suber
RJR-Nabisco
United States of America

Ian P. Tansey
3M Health Care 
United Kingdom

Adam Wanner
University of Miami
United States of America

Ashley Woodcock
Wythenshawe Hospital
United Kingdom

Hua Zhangxi
Ministry of Light Industry
China

* Corresponding members


TEAP Economic Options Committee
Chairs
Affiliation
Country     
Robert Van Slooten
Department of Trade and Industry
United Kingdom

Members
Affiliation




Country
Yusuf Ahmad
Consultant
Bangladesh

Penelope Canan
University of Denver
United States of America

Suely Carvalho
University of Sao Paulo
Brazil

Stephen DeCanio
University of California at Santa Barbara
United States of America

Mavis Holmes-Hanek
Ministry of Health and Environment
Bahamas

Ahmed Amin Ibrahim
Academy of Science Research and Technology
Egypt

Peter Landymore
Overseas Development Administration
United Kingdom

Anil Markandya
Harvard Institute for International Development
United States of America

Mesahiro Miyazaki
Ministry of International Trade and Industry (MITI)
Japan

David O'Connor
OECD Development Centre
France

Sergio Oxman
The World Bank

Bai Xianhong
China International Science Centre
China


TEAP Flexible and Rigid Foams Technical Options Committee
Chairs
Affiliation
Country
Jean Lupinacci
Environmental Protection Agency
United States of America

Sally Rand
Environmental Protection Agency
United States of America

Members
Affiliation
Country
Godfrey Abbott
Dow Europe/Exiba
Switzerland

Paul Ashford
BP Chemicals Ltd/EPFA
United Kingdom

Lorraine Aulisio
Celotex Corporation/PIMA
United States of America

Marion Axmith
The Society of the Plastics Industry (SPI)
Canada

Craig Barkhouse
Foamex Canada/CFFMA
Canada

Gert Baumann
Miles     
United States of America

Ted Biermann
BASF Corporation
United States of America

Michael J. Cartmell
ICI Polyurethanes
United States of America

John Clinton
NRG Barriers/PIMA
United States of America

Hubert Creyf
Recticel/Europur
Belgium

Shi Jia Fan
Qindao Haier Group Co.
China

Alan Fine
Environmental Protection Agency
United States of America

Ryoichi Fujimoto
Hitachi Limited
Japan

Reg Hurd
British Rubber Manufacturers Association
United Kingdom

Mike Jeffs
ICI Polyurethanes
Belgium

Robert Johnson
Whirlpool
United States of America

Fran Lichtenberg
The Society of the Plastics Industry (SPI)
United States of America

Yehia Lotfi
Technocom
Egypt

John Minsker
Dow Chemical
United States of America

Muneharu Sanoh
Japanese Electrical Manufacturers' Association
Japan

M. Sarangapani
Polyurethane Council of India
India

Ian Shankland
Allied Signal
United States of America

Sodario Souto
Brastemp S.A.
Brazil

Bert Veenendaal
RAPPA      
United States of America

Udo Wenning
Bosch-Siemens Hausgerate
Germany

Takao Yamamoto
Japan Urethane Foam Industrial Association
Japan


TEAP Halons Technical Options Committee
Chairs
Affiliation
Country
Tom Morehouse
Institute for Defense Analyses
United States of America

Gary Taylor
Taylor/Wagner     
Canada

Members
Affiliation
Country
David Ball
Kidde Graviner Limited
United Kingdom

Hervé Bineau
CTFHE
France

Walter Brunner
Envico AG
Switzerland

David Catchpole
BP Exploration (Alaska)
United States of America

Tom Cortina
Halon Alternatives Research Corporation
United States of America

Robert Darwin
Department of the Navy
United States of America

Philip DiNenno
Hughes Associates
United States of America

Ding Kangsheng
Zhejiang Chemical Industry Research Institute
China

Chris Hanauska
Hughes Associates
United States of America

H.S. Kaprwan
Defence Institute of Fire Research
India

Maj. Gen. Kataria
Defence Institute of Fire Research
India

Takaaki Konno
Fenwal Controls of Japan
Japan

Nikolai Kopylov
All Russian Research Inst. for Fire Protection
Russian Federation


Barbara Kucnerowicz-Polak
State Fire Services Headquarters
Poland

Arthur Lim
Institute of Fire Engineers
Singapore

Yvon Marty
CTFHE
France

Michelle Maynard
NASA
United States of America

Marion McQuaide
Ministry of Defence
United Kingdom

Mohamad Rodzi Sulaiman
Fire Services Department
Malaysia

John O`Sullivan
British Airways
United Kingdom

Erik Pedersen
Danish Fire Protection Association
Denmark

Gennadi Ryzhov
All Russian Research Inst. for Fire Protection
Russian Federation

Joseph Senecal
Fenwal Safety Systems 
United States of America

Ronald Sheinson
Naval Research Laboratory
United States of America

Robert E. Tapscott
NMERI
United States of America

Tony Thornhill
Department of National Defence
Canada

Daniel Verdonik
Department of the Army
United States of America

Brian Ward
Kidde Fire Protection
United Kingdom

Michael Wilson
Wormald Fire Systems
Australia

Roy Young
Loss Prevention Council
United Kingdom

Zhu Hailin
Tianjin Fire Research Inst.
China


TEAP Methyl Bromide Technical Options Committee
Chair
Affiliation
Country
Jonathan Banks
Commonwealth Scientific and Industrial Research 
         


  Organization (CSIRO)
Australia

Vice Chair
Affiliation
Country
Rodrigo Rodriguez-Kabana
Auburn University
United States of America

Members
Affiliation




Country


Joel arap-Lelei
Embassy of Kenya, Netherlands
Kenya

Mohd. Azmi Ab Rahim
Ministry of Agriculture
Malaysia

Thomas A. Batchelor
ENZA N.Z. (Intl.)
New Zealand 


Antonio Bello
Centro de Ciencias Medioambientales
Spain

Barry Blair
Tobacco Research Board
Zimbabwe

Richard C. Bruno
Sun Diamond Growers of California
United States of America

Adrian Carter
Agriculture Canada
Canada

Vicent Cebolla
Instituto Valenciana de  Investigaciones Agrarias 
Spain

Bishu Chakrabarti
Central Science Laboratory
United Kingdom

Chamlong Chettanachitara
Department of Agriculture
Thailand

Patricia Clary
Californians for Alternatives to Toxics/


  SAFE Alliance
United States of America 

Jorge Corona
CANACINTRA
Mexico

Miguel Costilla
Agro-Industrial Obispo Colombres
Argentina

Jennifer Curtis
Natural Resources Defense Council
United States of America

Tom Duafala
TriCal
United States of America

Patrick Ducom
Ministere de l'Agriculture et de la Peche
France 

Joe Eger
Dow Elanco
United States of America

Juan Francisco Fernandez
Ministero de Agricultura
Chile

Michael Graber
Ministry of the Environment
Israel

Avi Grinstein
Laboratory for Pesticide Application
Israel

Doug Gubler
University of California
United States of America

Joop van Haasteren
Ministry of Housing, Spatial Planning and 
Netherlands


  Environment

Thorkil E. Hallas
Danish Technological Institute
Denmark

Toshihiro Kajiwara
Japan Plant Protection Association
Japan

Jaacov Katan
Hebrew University
Israel

Richard Kramer
National Pest Control Association
United States of America

Laurent Lenoir
UCB SA
Belgium

Maria Ludovica Gullino
University of Turin
Italy

Michelle Marcotte
Nordion International     
Canada

Melanie Miller
S.A.F.E. Alliance
United Kingdom

Takamitsu Muraoka
Sanko Chemical Co.
Japan

Maria Nolan
Department of the Environment
United Kingdom

Joe Noling
University of Florida
United States of America

Henk Nuyten
Experimental Garden Breda
Netherlands

Gary Obenauf
Agricultural Research Committee
United States of America

Mary O'Brien
University of Montana
United States of America

David Okioga
Agricultural Research Institute
Kenya

William Olkowski
Bio-Integral Resource Center
United States of America

Sergio Oxman
The World Bank

Santiago Pocino
FMC Foret SA
Spain

Michael Host Rasmussen
Ministry of Environment
Denmark

A. Nathan Reed
Stemlit Growers     
United States of America

Christoph Reichmuth
Federal Biology and Research Center for


  Agriculture Forestry
Germany

Ralph Ross
Department of Agriculture
United States of America

Tsuneo Sakurai
Tiejin Chemicals 
Japan

John Sansone
SCC Products
United States of America

Colin Smith
Rentokil 
United Kingdom

Don Smith
Industrial Research 
New Zealand

Michael Spiegelstein
Bromine Compounds 
Israel

Morkel Steyn
Department of Health 
South Africa

Robert Suber
RJR Nabisco
United States of America

Akio Tateya
Agricultural Chemicals Inspection Station, MAFF
Japan

Robert Taylor
Natural Resources Institute
United Kingdom

Bill Thomas
Environmental Protection Agency
United States of America

Gary Thompson
Quaker Oats
United States of America

Jorn Tidow
BASF
Germany

Patrick Vail
Department of Agriculture-ARS
United States of America

Etienne van Wambeke
Katholieke Universiteit Leuven
Belgium

Kenneth Vick
Department of Agriculture
United States of America 

Chris Watson
IGROX 
United Kingdom

Robert Webb
Driscoll Strawberry Associates     
United States of America

Rene Weber
Great Lakes Chemical Co.
United States of America

James Wells
California Environmental Protection Agency
United States of America 

Wang Wenliang
Zhejiang Chemical Industry Research Institute
China 

Frank V. Westerlund
California Strawberry Advisory Board 
United States of America


TEAP Refrigeration, Air Conditioning and Heat Pumps 


Technical Options Committee
Chair
Affiliation
Country
Lambert Kuijpers
Technical University
Netherlands 

Section Chairs
Affiliation
Country
Rune Aarlien
SINTEF
Norway

R.S. Agarwal
Indian Institute of Technology
India

Ward Atkinson
Sun Test Engineering
United States of America

James A. Baker
Harrison Division, General Motors
United States of America

Jos W. Bouma
IEA Heat Pump Center
Netherlands

Denis Clodic
Ecole des Mines
France

Peter Cooper
Adtec Services
United Kingdom

Jim Crawford
Trane Co.
United States of America

David Didion
NIST
United States of America

Robert Heap
SRCRA
United Kingdom

Hans Haukas
Refrigeration Consultant
Norway

Kenneth Hickman
York International Co.
United States of America

Fred Keller
Carrier 
United States of America

Louis Lucas
IIR
France

Kenneth W. Manz
Robinair, SPX Corporation
United States of America

Edward J. McInerney
General Electric
United States of America

Mark O. McLinden
NIST 
United States of America

Mark Menzer
ARI
United States of America

S. Forbes Pearson
Star Refrigeration
United Kingdom

Frederique Sauer
Dehon Service SA
France

Erik Schau
UNITOR Ships Service
Norway

Sonny Sundaresan
Copeland Co.
United States of America

Paulo Vodianitskaia
Multibras SA 
Brazil

Lau Vors
L&E Teknik og Management
Denmark

Members
Affiliation
Country
Kent Anderson
Int. Inst. of Ammonia Refrigeration
United States of America

Gianfranco Angelino
Politecnico di Milano
Italy

David Bateman
E.I. DuPont de Nemours and Company
United States of America

Russell Benstead
EA Technology
United Kingdom

Angelo Bertu
Whirlpool
Italy

S.C. Bhaduri
Indian Institute of Technology
India

Donald B. Bivens
E.I. DuPont de Nemours and Company
United States of America

Paul Brauch
Vilter Manufacturing 
United States of America

James M. Calm
Engineering Consultant
United States of America

Douglas Cane
Caneta Research     
Canada

Terry Chadderton
Meat Industry Research Inst.
New Zealand

Joel Crespin
Unite Hermetique
France

Mark Cywilko
Carrier Transicold
United States of America

Per O. Danig
Technical University 
Denmark

Mohamed Zahoor Dean
Remco 
Kenya

Sukumar Devotta
National Chemical Lab.
India

Jan Duiven
AEER
Belgium

Richard Ertinger
Carrier 
United States of America

Yu Bing Feng
Xi'an Jiatong University
China

David Gibson
W.S. Atkins Energy 
United Kingdom

Herbert T. Gilkey
Engineering Consultant
United States of America

Poul-Erik Hansen
Danfoss GmbH
Germany

Laercio Hardt
Embraco S/A
Brazil

John Hatton
Sea Containers
United Kingdom

Ulrich Hesse
Spauschus Associates
United States of America

Shunya Hisashima
JRAIA
Japan

Sachio Hotani
Japanese Association of Refrig.
Japan

Glen Hourahan
ARI
United States of America

Michael Hughes
Allied Signal      
United States of America

Y. Igarashi
Heat Pump Technology Center
Japan

Yukinobo Ikemoto
Mitsubishi Heavy Industries
Japan

Martien Janssen
Re/genT Co.
Netherlands

Werner Jensen
Integral Technologie
Germany

Ren Jinlu
GMRI
China

James F. Kanyua
University of Nairobi
Kenya

Yasuhiro Kawanishi
Sanyo
Japan

Pieter Koelet
NV Schatten SA
Belgium

Bill Kopko
Environmental Protection Agency
United States of America

Erik Korfitsen
Sabroe Refrigeration A/A
Denmark

Horst Kruse
University of Hannover
Germany

Haw En Kwi
Nippon Denso
Malaysia

Harold Lamb
Atochem North America
United States of America

H.J. Laue
Fachinform. Karlsruhe GmbH
Germany

Laurent Legin
Societe Trane
France

Peter Likes
Hussman Co. 
United States of America

Anders Lindborg
Frigoscandia AB
Sweden

Hugh McDonald
Ministry of Defence
United Kingdom

Katharine Miller
Battelle PNL Labs
United States of America

Yoshiyuki Morikawa
Matsushita Electric 
Japan

Peter Moser
Sulzer Friotherm 
Switzerland

Roland Mottal
IIR
France

Gale Myers
Gas Research Institute
United States of America

M. Narodoslawsky
Graz University of Technology
Austria

M. Nonnenmann
Behr & Co. GmbH
Germany

Lars Nordell
LGN - Energikonsult
Sweden

Richard Oas
Safeway     
United States of America

Tomishige Oisumi
Toshiba 
Japan

Robert Orfeo
Allied Signal      
United States of America

Deborah Ottinger
Environmental Protection Agency
United States of America

Cristophe Petitjean
VALEO
France

E. Preisegger
Hoechst AG
Germany

Chuck Purcell
Battelle PNL Labs
United States of America

K. Rao
Kelvinator of India
India

George Redden
Dunhan-Bush     
United States of America

Wilhelm Ritter
Upper-Austrian Electric Power Co.
Austria

Lindsey Roke
Fisher and Paykel
New Zealand

Kazuo Sahara
Daikin Industries
Japan

Norio Sawada
Sanyo Co.
Japan

Rajendra Shende
UNEP IE/PAC
France

Arnon Simakulthorn
Thai Compressor Ltd
Thailand

John Smale
Environment Canada
Canada

Leong Kam Son
York International
Malaysia

Rich Sweetser
Gas Cooling Center
United States of America

Alan Tang
Sanden AC
Malaysia

Reiner Tillner-Roth
University of Hannover
Germany

Lennert Vamling
Chalmers University
Sweden

Ed Vineyard
Oak Ridge National Lab
United States of America

Tony Vogelsberg
E.I. DuPont de Nemours and Company
United States of America

Tom Waltz
The World Bank

Koichi Watanabe
Keio University
Japan

Peirre Weiss
Elf-Atochem
France

A. Wilson
Lloyds Register of Shipping
United Kingdom

Kiyoshige Yokoi
Matsushita Refr. Co.
Japan

Ming Shan Zu
Tsinghua University
China


TEAP Solvents, Coatings and Adhesives


Technical Options Committee
Chair
Affiliation
Country
Stephen O. Andersen
Environmental Protection Agency
United States of America

Vice-Chair
Affiliation
Country
Jorge Corona
CANACINTRA
Mexico


TEAP Solvents, Coatings and Adhesives


Technical Options Committee
Members
Affiliation
Country
Husamuddin Ahmadzai
Environmental Protection Agency
Sweden

Lorenzo Alvarez
South America Electronics Operation
Brazil


David Andrews
GEC Marconi Hirst
United Kingdom

Jay Baker
Ford
United States of America

Bryan Baxter
British Aerospace
United Kingdom

Charles Carpenter
Waste Policy Institute
United States of America

Pakasit Chanvinij
Thai Airways
Thailand

Mike Clark
Sketchley
United Kingdom

Brian Ellis
Protonique
Switzerland

Stephen Evanoff
Lockheed Corporation Fort Worth
United States of America

Joe Felty
Texas Instruments
United States of America

John Fisher
AT&T
United States of America

Art FitzGerald
IFC Northern Telecom
Canada

Pamela Foster
Friends of the Earth
Canada

Yuichi Fujimoto
Japan Electrical Manufacturers' Association
Japan
 

G. Gabelmann
ITT Teves
Germany

Leslie Guth
AT&T 
United States of America

Don Hunt
Air Force
United States of America

Yoshiyuki Ishii
Hitachi
Japan

Peter Johnson
European Chlorinated Solvents Association
United Kingdom

William Kenyon
Global Centre for Process Change
United States of America

Sudhakar Kesavan
ICF
United States of America

Hiroshi Kurita
Japan Association for Hygiene of Chlorinated 


  Solvents
Japan

Stephen Lai
Singapore Inst. of Standards and Industrial 


  Research
Singapore

Leo Lambert
Digital Equipment Corporation
United States of America

Milton Lubraico
Ford
Brazil

Mohinder Malik
Lufthansa German Airlines
Germany

Shigeo Matsui
Japan Audit and Certification Organization
Japan

Annie Maurel-Groleau
Telemecanique
France

James Mertens
Dow 
United States of America

Hank Osterman
Allied Signal     
United States of America

Fritz Powolny
OXITENO
Brazil

Cynthia Pruett
Consultant 
United States of America

Patrice Rollet
Promosol
France

Wolf-Eberhard Schiegl
Siemens
Germany

Hussein Shafa'amri
Ministry of Planning
Jordan

John Shirtz
Air Force
United States of America

Darrel Staley
Boeing
United States of America

John Stemniski
Charles Stark Draper Labs
United States of America

Doug van Mullem
Air Force
United States of America

John Wilkinson
Vulcan Materials
United States of America

Masaaki Yamabe
Asahi Glass
Japan

X`Avier HK Yoong
National Semiconductor
Malaysia


TEAP Destruction Technology SubCommittee
Chair
Affiliation
Country
Abe Finkelstein
Environment Canada
Canada

Members
Affiliation
Country
Godfrey Abbott
EXIBA
Switzerland

Stephen Andersen
Environmental Protection Agency
United States of America

Tom Bell
Simon Fraser University
Canada

Jerry Brown
Allied Defense Industries
United States of America

Nick Campbell
ICI Chemicals
United Kingdom

Paul Cammer
Cammer and Associates
United States of America

Don Colley
Bovar     
Canada

Brent Davey
Centre for Environmental Mgt.
Australia

Dave Davis
Vulcan Chemicals
United States of America

James DeAngelis
Commodore Environmental Services
United States of America

Connie Deford
Dow Chemical
United States of America

Vinci Felix
E.I. DuPont de Nemours and Company
United States of America

Howard Greene
University of Akron
United States of America

Robert Hall
Environmental Protection Agency
United States of America

Kirk Hummel
Radian 
United States of America

Chun Wai Lee
Environmental Protection Agency
United States of America

Karen Metchis
Environmental Protection Agency
United States of America

Koichi Mizuno
MITI
Japan

Maurice Oubre
Dow Chemical
United States of America

Kash Ram
Environment Canada
Canada

Rosemary Townsend
Commonwealth EPA
Australia


TEAP Inadvertent Losses SubCommittee
Chair
Affiliation
Country
Nick Campbell
ICI Chemicals and Polymers Ltd 
United Kingdom

Lambert Kuijpers
Technical University Eindhoven 
Netherlands

Members
Affiliation
Country
Kathi Anderson
E.I. DuPont de Nemours and Company 
Canada

Sandip Bhatia
Navin Fluorine Industries 
India

Connie Deford
Dow Chemicals (North America)
United States of America

Kevin Fay
Alliance for Responsible Atmospheric Policy
United States of America

Jean Lamont
Department of Trade and Industry 
United Kingdom

Paul Horwitz
Environmental Protection Agency 
United States of America

Hiroshi Kurita
Japanese Association for the Hygiene of 


  Chlorinated Solvents
Japan

Vivian McIntire
Eastman Chemical Company 
United States of America

Wolfgang Scholten
Hoechst AG 
Germany

Y Umeki
Mitsui-DuPont
Japan

John Wilkinson
Vulcan Chemicals 
United States of America

Xiao Xu Pei
Shanghai Institute of Organo Fluorine Materials
China 


-----  

     �  The December 1994 Technology and Economic Assessment Panel (TEAP) report is based on reports of the seven Technical Options Committees:  Aerosol Products, Sterilants, Miscellaneous Uses and Carbon Tetrachloride;  Economic Options;  Flexible and Rigid Foams;  Halons;  Methyl Bromide;  Refrigeration, Air-Conditioning and Heat Pumps;  and Solvents, Coatings and Adhesives.


     �  There were separate panels for the technology and economic assessments in 1989 but they were combined in 1991 and subsequent years.


     �  In Decision VI/13 the Parties requested the Assessment Panels to report by March 1995 on the feasibility and implications of the available alternatives and substitutes to HCFC and methyl bromide.  





     �  It appears from initial observations that the Antarctic ozone "hole" in 1994 was comparable to that of 1992-1993.





